Compact, economical and flexible

New diffuse-reflection sensor with background suppression of ipf

With the OTQ80576, ipf has added another optical diffuse-reflection sensor in a compact format with a width of 8mm and a height of 21mm to its range, which is an economical alternative to comparable devices in a larger design.

The sensor equipped with a PIN point LED (wavelength 644nm) generates a small red light spot. The sensing range is adjustable from 3mm to 132mm. Furthermore, the diffuse reflection sensor is equipped via background suppression and thus enables reliable detection that is virtually independent of the color and surface of an object.

The sensing range is set either via teach-in or IO-Link. Both a dynamic mode for detecting moving objects and a static mode for detecting stationary objects are available for teach-in. Thanks to the IO-Link interface, the OTQ80576 can also be parameterized very flexibly to meet the respective requirements of an application. This includes various hysteresis settings, the setting of a time delay or pulse stretching, a counter with reset function and the choice between a limit or window function during operation, to name just a few examples. In addition, the measured value quality and thus the reflectivity of the optical diffuse reflection sensor in a specific application can be viewed via IO-Link.

The OTQ80576 in robust industrial plastic housing (protection class IP67) is designed for ambient temperatures from -25°C to +60°C. The electrical connection is made using a four-pin M8 cable connector. A flexibly adjustable ball joint holder (AY000116) is also available for mounting.

Caption: Economical compact solution: The OTQ80576 optical diffuse-reflection sensor

with background suppression and IO-Link interface from ipf. (Image: ipf electronic gmbh)

ABOUT IPF ELECTRONIC

Sensors of the highest quality

When HIGH-TECH becomes HIGH-END

We have stood for high-performance sensors in automation technology in the German-speaking markets for over four decades. We prioritize the highest level of quality and have our own production at our headquarters in Altena in Sauerland.

We are ipf electronic and do more than just our job. We think outside the box, create innovative, sustainable solutions, and remain approachable. We are based in Sauerland, one of the most innovative areas in Germany. Our products are precise, intelligent, technologically well-engineered and versatile. Our 140 employees live and breathe service, even outside of normal business hours.

Our wide range of products, great problem-solving skills and strong focus on service make us a unique top-supplier of industrial sensor technology. Permanent research and development play an equally substantial role as the education and training of employees and management. Our company, which was founded in 1982, is managed today by the family in the second generation. We apply special standards in environmental protection and sustainable resource management.

CONTACT

ipf electronic gmbh

Rosmarter Allee 14

58762 Altena

Germany

info@ipf-electronic.com

www.ipf-electronic.com

PRESS CONTACT

Martinus Menne

Waldweg 8 ● 57489 Drolshagen

Germany

Tel +49 2761 8288861

mm@technikredaktion.de

www.technikredaktion.de