Much more than contactless linear measurement

New parameterizable system solution from ipf

With the VO330570, ipf presents an optical displacement encoder that can do far more than its name suggests.

The incremental sensor with a sensing range of 15 to 60mm detects all moving objects contactless and therefore without slippage,

regardless of material or color. As the device works with infrared light of laser class 1, no special safety precautions are required during installation. Due to the very high sensing rate of 0.9 milliseconds, the VO330570 is particularly suitable for detecting materials in very

fast-moving processes. But the solution can do much more.

The sensor is parameterized via free PC software, with the units pulse/mm, m/min and mm available. If the VO330570 remains connected to the PC during operation, the software also determines the traverse speed of the detected object. The solution from ipf also integrates an encoder output that provides two 90-degree phase-shifted square wave signals A/B, so that a travel distance can also be displayed through the connection of a counter.

Via a further switching output, it is also possible to parameterize a zero point for a predefined distance (e.g. every 50mm) in order to reset a counter, for example, or to process the signal in a PLC (programmable logic controller), e.g. to control automated cutting of strip materials. The second output can also be parameterized as an alarm output, which significantly increases the process reliability of the VO330570's displacement and angular measurements.

Caption: Contactless displacement and angular measurement, display of traverse speed or configure travel distance,

zero point or alarm output - The optical sensor VO330570 from ipf in an aluminum housing offers enormous flexibility of use.

(Image: ipf electronic gmbh)

Caption: Flexible solution with great potential: Use of the VO330570 for contactless,

slip-free and surface-independent linear measurement. (All images: ipf electronic gmbh)

ABOUT IPF ELECTRONIC

Sensors of the highest quality

When HIGH-TECH becomes HIGH-END

We have stood for high-performance sensors in automation technology in the German-speaking markets for over four decades. We prioritize the highest level of quality and have our own production at our headquarters in Altena in Sauerland.

We are ipf electronic and do more than just our job. We think outside the box, create innovative, sustainable solutions, and remain approachable. We are based in Sauerland, one of the most innovative areas in Germany. Our products are precise, intelligent, technologically well-engineered and versatile. Our 140 employees live and breathe service, even outside of normal business hours.

Our wide range of products, great problem-solving skills and strong focus on service make us a unique top-supplier of industrial sensor technology. Permanent research and development play an equally substantial role as the education and training of employees and management. Our company, which was founded in 1982, is managed today by the family in the second generation. We apply special standards in environmental protection and sustainable resource management.

CONTACT

ipf electronic gmbh

Rosmarter Allee 14

58762 Altena

Germany

info@ipf-electronic.com

www.ipf-electronic.com

PRESS CONTACT

Martinus Menne

Waldweg 8 ● 57489 Drolshagen

Germany

Tel +49 2761 8288861

mm@technikredaktion.de

www.technikredaktion.de