More than two birds with one stone

Multifunctional inductive ring sensors from IPF

If you want to reliably recognize metal objects in industry, even under extremely harsh operating conditions, the best solution is to use the wide range of inductive sensors from IPF. But what to do if a small, metallic component ejected from a machine does not hit the detection range of the sensor? The answer is provided by the new multifunctional inductive ring sensors from IPF.

The active area of these sensors is located within a ring-shaped opening. If a metal part falls through, the alternating magnetic field of the active surface is damped and a switching signal is triggered. For example, metallic objects can be passed through a plastic hose that is guided through the opening of the sensor to reliably detect the objects.

When detecting metallic parts with inductive ring sensors, a general distinction is made between static and dynamic modes of operation. Static: The switching output remains active as long as an object is in the active zone (ideal e.g. for fill level or accumulation monitoring or presence monitoring). Dynamic: A metal part must move, whereby the switching output is set for a certain time when the object is detected until the active zone is free again (ideal e.g. for countering parts). However, separate devices were previously required for these two operational modes.

This has now changed with the new family of inductive ring sensors from IPF. With just one device version, teach-in can be used to select not only the sensitivity, but also the desired operational mode (static or dynamic) of the probes and between a normally closed and normally open function for their switching output.

Furthermore, the integrated IO-Link interface enables a whole range of additional, helpful settings, e.g. the choice between different device modes (standard, precision, speed), turn-off delay of the switching output, hysteresis setting between 0 and 20%, single value, two-value or window function for the switching output, additional virtual switching output when operating with an IO-Link-capable control (unit), single-point, two-point or dynamic teaching for each switching output, etc. The additional options available via IO-Link can be used, for example, to determine the drop speed of objects in the detection range of the sensor or to set the detection of metal parts of a specific size or size range, to name just a few practical examples.

Apart from the housing color (black instead of the previous blue), the new devices are identical in construction to the previous inductive ring sensors from IPF. As the sensitivity can also be adjusted as usual via a potentiometer, there is nothing to stop you replacing previously installed IPF sensors without any problems.

The new series is available with ring diameters between 6mm and 50mm. New is the IY300520, which closes the gap between the IY250520 (Ø 25mm) and the IY350520 (Ø 35mm) with a ring diameter of 30mm. The static (IYK00320) and dynamic (IYK00325) versions of the sensors with a ring diameter of 100mm from the previous series remain part of the IPF range.

Caption: The new inductive ring sensors with IO-Link interface are convincing

with their freely selectable operational modes and high flexibility of use.

(All images: ipf electronic gmbh)

IPF ELECTRONIC AT SPS:
HALL 7A, BOOTH 131

ABOUT IPF ELECTRONIC

Sensors of the highest quality

When HIGH-TECH becomes HIGH-END

We have stood for high-performance sensors in automation technology in the German-speaking markets for over three decades. We prioritize the highest level of quality and have our own production at our headquarters in Altena in Sauerland.

We are ipf electronic and do more than just our job. We think outside the box, create innovative, sustainable solutions and remain approachable. We are based in Sauerland, one of the most innovative areas in Germany. Our products are precise, intelligent, technologically well-engineered and versatile. Our 140 employees live and breathe service, even outside of normal business hours.

Our wide range of products, great problem-solving skills and strong focus on service make us a unique top-supplier of industrial sensor technology. Permanent research and development play an equally substantial role as the education and training of employees and management. Our company, which was founded in 1982, is managed today by the family in the second generation. We apply special standards in environmental protection and sustainable resource management.

CONTACT

ipf electronic gmbh

Rosmarter Allee 14

58762 Altena

Germany

info@ipf-electronic.com

www.ipf-electronic.com

PRESS CONTACT

Martinus Menne

Waldweg 8 ● 57489 Drolshagen

Germany

Tel +49 2761 8288861

mm@technikredaktion.de

www.technikredaktion.de