Seeing instead of guessing

New inclination sensors from IPF

With the YN58-series, IPF is presenting inclination sensors that are particularly impressive due to their "inner values".

The sensors detect the inclination angle in both the X and Y directions with an absolute repeat accuracy of 0.03°. The sensors are available in three versions: -10° to +10° (YN580020), -45° to +45° (YN580021) and -85° to +85° (YN580022), with the two analog outputs providing a signal of 4...20mA via the entire measuring range.

Compared to other inclination sensors on the market, the YN58-series from IPF has some special features. For example, there is an easy-to-read LED display inside the sensors to visualize the inclination angle. The information provided by the LED display is also supported by four crosshair LEDs arranged around the display, which signal the direction in which the sensor is tilted during installation. Both the display and the crosshair LED thus significantly simplify the installation and commissioning of the devices.

The housing of the inclination sensors, designed for a wide ambient temperature range from -25°C to +80°C, is made of aluminum (protection class IP67). The electrical connection is made using a 5-pin M12-connector.

Inclination sensors are used for absolute angle and position detection and measure the angle of an object in relation to the earth gravity. The range of applications for such devices is diverse: e.g. on excavators, cranes, lifting platforms (accident protection), PV systems, wind turbines, telescopes or industrial robots, to name just a few examples.

Caption: The new inclination sensors from IPF are particularly impressive due to their "inner values",

as they integrate both an LED display and four crosshair LEDs.

(All images: ipf electronic gmbh)

IPF ELECTRONIC AT SPS:
HALL 7A, BOOTH 131

ABOUT IPF ELECTRONIC

Sensors of the highest quality

When HIGH-TECH becomes HIGH-END

We have stood for high-performance sensors in automation technology in the German-speaking markets for over three decades. We prioritize the highest level of quality and have our own production at our headquarters in Altena in Sauerland.

We are ipf electronic and do more than just our job. We think outside the box, create innovative, sustainable solutions and remain approachable. We are based in Sauerland, one of the most innovative areas in Germany. Our products are precise, intelligent, technologically well-engineered and versatile. Our 140 employees live and breathe service, even outside of normal business hours.

Our wide range of products, great problem-solving skills and strong focus on service make us a unique top-supplier of industrial sensor technology. Permanent research and development play an equally substantial role as the education and training of employees and management. Our company, which was founded in 1982, is managed today by the family in the second generation. We apply special standards in environmental protection and sustainable resource management.

CONTACT

ipf electronic gmbh

Rosmarter Allee 14

58762 Altena

Germany

info@ipf-electronic.com

www.ipf-electronic.com

PRESS CONTACT

Martinus Menne

Waldweg 8 ● 57489 Drolshagen

Germany

Tel +49 2761 8288861

mm@technikredaktion.de

www.technikredaktion.de