Space saver for the top-hat rail

New IO-Link logic module from IPF

IPF is one of the pioneers in the field of logic modules, as the sensor specialist from the Sauerland region developed one of the first and still sought-after solutions for DIN rail mounting for the logical linking of sensor signals in the field decades ago with the VL25-series devices. IPF also recognized the potential of IO-Link for flexible parameterization of logic modules at a very early stage. The logical consequence: with the VL530100, IPF is now presenting its first highly flexible IO-Link logic module for freely wireable control cabinet mounting.

Many advantages of decentralized signal processing

Modern production processes cannot do without a large number of sensors for a wide variety of tasks. However, the straight inquiry of many sensors via a system controller (PLC) can prolong cycle times considerably and, via this, increases the need for inputs and signal lines. Decentralized processing of sensor signals with logic modules, on the other hand, significantly reduces the load on a PLC, ensures shorter cable runs, reduces the wiring effort and also reduces the need for control inputs.

A particularly slim and flexible solution

The VL530100 IO-Link logic module requires an operating voltage of 10-30V DC and, with a width of just 22.5mm, has been specially designed to save space on the top hat rail. The VL530100 is similar to its siblings in the VL31 and VL61-series with an IO-Link interface in terms of its range of features, although the functions of the new module are identical to those of IPF's VL610328 IO-Link logic module. The two digital switching outputs of the VL530100 can be logically linked with up to eight sensor inputs (AND/OR operation). Both the inputs and logics can be freely selected.

Proven in practice: Virtual groups and debounce function

The VL530100 also incorporates some tried-and-tested features from IPF's previous IO-Link logic modules. These include the creation of so-called virtual groups, which allow several conventional logic distributors to be replaced by a single IO-Link logic module.

Another feature is a so-called debounce function. This makes it possible to route a signal via a mechanical contact.  If the signal of a mechanical limit switch, plunger switch or roller limit switch is to be coupled to a turn of the VL530100 instead of a sensor signal, this special function prevents the evaluation of several switching signals due to contact bounce of a switch.

Caption: Another milestone in IO-Link logic modules from IPF:

The new, very slim VL530100 for AND/OR connections was specially developed for

developed for mounting on a top hat rail

(All images: ipf electronic gmbh)

IPF ELECTRONIC AT SPS:
HALL 7A, BOOTH 131

ABOUT IPF ELECTRONIC

Sensors of the highest quality

When HIGH-TECH becomes HIGH-END

We have stood for high-performance sensors in automation technology in the German-speaking markets for over three decades. We prioritize the highest level of quality and have our own production at our headquarters in Altena in Sauerland.

We are ipf electronic and do more than just our job. We think outside the box, create innovative, sustainable solutions and remain approachable. We are based in Sauerland, one of the most innovative areas in Germany. Our products are precise, intelligent, technologically well-engineered and versatile. Our 140 employees live and breathe service, even outside of normal business hours.

Our wide range of products, great problem-solving skills and strong focus on service make us a unique top-supplier of industrial sensor technology. Permanent research and development play an equally substantial role as the education and training of employees and management. Our company, which was founded in 1982, is managed today by the family in the second generation. We apply special standards in environmental protection and sustainable resource management.

CONTACT

ipf electronic gmbh

Rosmarter Allee 14

58762 Altena

Germany

info@ipf-electronic.com

www.ipf-electronic.com

PRESS CONTACT

Martinus Menne

Waldweg 8 ● 57489 Drolshagen

Germany

Tel +49 2761 8288861

mm@technikredaktion.de

www.technikredaktion.de