Pressure sensors for temperatures from -40°C to +100°C

New devices from IPF with IO-Link interface

With the EMB50171 IPF presents a ring light that has been specially developed for the spindle holder of a machining center.

The DW25-series pressure sensors from IPF impress above all with their exceptionally wide media temperature range from

-40 °C to +100 °C. Due to their measuring ranges of -0.1bar...0.1bar (DW25310N) and 0bar...0.25bar (DW25310P), the devices are suitable for particularly demanding environments and are ideal for static level measurements in containers with liquids, e.g. hydraulic oils, cooling lubricant emulsions, etc. or as 2-point controllers for controlling pumps.

Both devices with G1/4" process connection have two outputs, a pure switching output and a second output whose function is variably switchable via IO-Link as switching, analog (0...20mA, 4...20mA) or alarm output. The IO-Link interface also allows additional demand-

specific, fully customizable parameterization, e.g., simple switchover from normally open to normally closed, adjustment of switching and release points, hysteresis settings, window functions, definition of turn-on/turn-off delays, storage of minimum and maximum values or damping settings. For example, damping can be used to smooth the measured values in applications with very dynamic pressure fluctuations.

With the DW25, IPF is expanding its portfolio of compact pressure sensors, which are designed for all gaseous and liquid media (non-hazardous, non-flammable, non-toxic) as "all-rounders".

Caption: DW25-series pressure sensors in stainless steel housing (IP67) withstand media

media temperatures from -40°C to +100°C.

(All images: ipf electronic gmbh)

IPF ELECTRONIC AT SPS:
HALL 7A, BOOTH 131

ABOUT IPF ELECTRONIC

Sensors of the highest quality

When HIGH-TECH becomes HIGH-END

We have stood for high-performance sensors in automation technology in the German-speaking markets for over three decades. We prioritize the highest level of quality and have our own production at our headquarters in Altena in Sauerland.

We are ipf electronic and do more than just our job. We think outside the box, create innovative, sustainable solutions and remain approachable. We are based in Sauerland, one of the most innovative areas in Germany. Our products are precise, intelligent, technologically well-engineered and versatile. Our 140 employees live and breathe service, even outside of normal business hours.

Our wide range of products, great problem-solving skills and strong focus on service make us a unique top-supplier of industrial sensor technology. Permanent research and development play an equally substantial role as the education and training of employees and management. Our company, which was founded in 1982, is managed today by the family in the second generation. We apply special standards in environmental protection and sustainable resource management.

CONTACT

ipf electronic gmbh

Rosmarter Allee 14

58762 Altena

Germany

info@ipf-electronic.com

www.ipf-electronic.com

PRESS CONTACT

Martinus Menne

Waldweg 8 ● 57489 Drolshagen

Germany

Tel +49 2761 8288861

mm@technikredaktion.de

www.technikredaktion.de