High-precision radar sensor for contactless level detection

New solution completes ipf electronic's comprehensive portfolio

ipf electronic has a comprehensive portfolio of sensor solutions for media-contacting and contactless level detection. The new FR900020 now completes this very wide range with a device with radar technology.

The FR900020, which is contactless and therefore wear-free, works according to the transit time method. The device periodically transmits a radar signal with a linearly increasing and decreasing frequency and a constant rate of change via time, which is reflected by a liquid

medium. The filling level in a container is then determined via the transit time shift and the frequency deviation of the reflected radar

signal.

The new FR900020 radar sensor from ipf electronic is a real alternative for applications in which optical sensors or ultrasonic sensors reach their limits due to environmental conditions such as temperature, gas or steam, positive or negative pressure or vacuum as well as dust and interfering ambient light. The radar sensor offers clear advantages over ultrasonic sensors, for example, when monitoring the level of media on which foam can form. In addition, the radar sensor also enables detection through a plastic lid, which serves as a container cover or protective window, for example.

The FR900020 in protection class IP67 and IP69k with M12-connector impresses with a very short response time of 0.03ms, a high range of up to 10m and a high focusing of the radar signal with an angle of beam spread of just 6°. The sensor with a housing made of stainless steel and chrome-nickel steel is insensitive to dirt or moisture and is designed for media temperatures from -50°C to +85°C.

Caption: The FR900020 radar sensor completes ipf electronic's comprehensive range

in the field of level control and, thanks to its properties, can be a real alternative to

optical sensors or ultrasonic sensors, for example.
(Image: ipf electronic gmbh)

ABOUT IPF ELECTRONIC

Sensors of the highest quality

When HIGH-TECH becomes HIGH-END

We have stood for high-performance sensors in automation technology in the German-speaking markets for over three decades. We prioritize the highest level of quality and have our own production at our headquarters in Altena in Sauerland.

We are ipf electronic and do more than just our job. We think outside the box, create innovative, sustainable solutions and remain approachable. We are based in Sauerland, one of the most innovative areas in Germany. Our products are precise, intelligent, technologically well-engineered and versatile. Our 140 employees live and breathe service, even outside of normal business hours.

Our wide range of products, great problem-solving skills and strong focus on service make us a unique top-supplier of industrial sensor technology. Permanent research and development play an equally substantial role as the education and training of employees and management. Our company, which was founded in 1982, is managed today by the family in the second generation. We apply special standards in environmental protection and sustainable resource management.

CONTACT

ipf electronic gmbh

Rosmarter Allee 14

58762 Altena

Germany

info@ipf-electronic.com

www.ipf-electronic.com

PRESS CONTACT

Martinus Menne

Waldweg 8 ● 57489 Drolshagen

Germany

Tel +49 2761 8288861

mm@technikredaktion.de

www.technikredaktion.de